KIPR Open Robot 2 Documentation

Table of Contents

KIPR Board3

Schematic3

Layout4

Control Systems5

Motor Controller5

PSoC8

Raspberry Pi 9

Board 9

Code Overview 9

Robot Overview 10

Mechanical 11

Electrical 12

Functionality 13

# KIPR Board

# PCB Schematic

Board Layout

Control Systems

Motor Controller

Top side

Bottom Side

The TB6612FNG motor driver can control up to two DC motors at a constant current of 1.2A (3.2A peak). Two input signals (IN1 and IN2) can be used to control the motor in one of four function modes - CW, CCW, short-brake, and stop. The two motor outputs (A and B) can be separately controlled, the speed of each motor is controlled via a PWM input signal with a frequency up to 100kHz. The STBY pin should be pulled high to take the motor out of standby mode.

Logic supply voltage (VCC) can be in the range of 2.7-5.5VDC, while the motor supply (VM) is limited to a maximum voltage of 15VDC. The output current is rated up to 1.2A per channel (or up to 3.2A for a short, single pulse).

The board can be ordered form Sparkfun Electronics, product SKU 09457.

The board pinout is as follows:

VCC: 5V

GND: Common GND

VM: Motor supply voltage

PWMA/B: Pulse width modulated signal for controlling speed.

A01/A02: Motor A connections

B01/B02: Motor B connections

AIN1/AIN2: Motor A control pins

BIN1/BIN2: Motor B control pins

STBY: Stops motors and places controller in low power state.

Control States:

HW Diagram

PSoC

The PSoC CY8C29466-24PXI is the brain of the control systems. It reads the encoders, communicates with the Motor Controller, and handles serial messages with the Raspberry Pi. There is a range of functions that the PSoC has been programmed to perform and they can all be accessed via serial messages from the Raspberry Pi.

PSoC Pinout:

Functional Description of Pins:

| Pin Name | Functional Description | Pin Name | Functional Description |
| --- | --- | --- | --- |
| Misc 0 | Allocated to Servo1 PWM module | VDD | 5V Supply |
| Misc 1 | Unused, available for use upon reprogramming | VSS | GND |
| Misc 2 | Allocated to Servo0 PWM module | ENC1A | A output of encoder1 |
| Misc 3 | Unused, available for use upon reprogramming | ENC1B | B output of encoder1 |
| Misc 4 | Unused, available for use upon reprogramming | ENC2A | A output of encoder2 |
| Misc 5 | Light Sensor input | ENC2B | B output of encoder2 |
| Misc 6 | Unused, available for use upon reprogramming | STDBY | Motor control STDBY, unused |
| Misc 7 | Debug pin, also available for reprogramming* | PWMA | Motor control PWMA, allocated to a PWM module |
| Misc 8 | Debug pin, also available for reprogramming* | PWMB | Motor control PWMB, allocated to a PWM module |
| RX | Serial receive line, goes to Raspberry Pi TX | AIN1 | Motor control AIN1, allocated to output pin |
| TX | Serial transmit line, goes to Raspberry PI RX through 2/3 divider | AIN2 | Motor control AIN2, allocated to output pin |
| PSCK | I2C clock, used for programming | BIN1 | Motor control BIN1, allocated to output pin |
| PSDA | I2C data, used for programming | BIN2 | Motor control BIN2, allocated to output pin |
| XRES | External Reset, used for programming | SMP | PSoC switch mode pump, unused |

*Debug pins can be repurposed without disrupting the debug functionality.

Debug Mode

Debug mode will allow the board to operate as normal, however every serial command sent will echo back useful debugging data. To enable debug mode connect each debug pin to +5V before powering the board.

Available serial commands:

| Command | Packet | Description |
| --- | --- | --- |
| MAV | "a<p>\a" | Set robot velocity to <p> RPS |
| MOV | "b<p>\a" | Set motor PWMs to pulse width <p> (8-bit) |
| GETV | "c\a" | Gets the current velocity of the robot in RPS |
| SERV0_POS | "d<p>\a" | Sets servo0 PWM to pulse width <p> (16-bit) |
| SERV1_POS | "e<p>\a" | Sets servo1 PWM to pulse width <p> (16-bit) |
| SRV0_STP | "f\a" | Stops servo0 PWM |
| SRV1_STP | "g\a" | Stops servo1 PWM |
| GETC1 | "h\a" | Get's the cumulative running count of encoder 1 |
| GETC2 | "i\a" | Get's the cumulative running count of encoder 2 |
| RSTC1 | "j\a" | Resets running count of encoder 1 to 0 |
| RSTC2 | "k\a" | Resets running count of encoder 2 to 0 |
| STOP | "l\a" | Stops the robot |
| FORWARD | "m\a" | Sets motor directions to give forward movement |
| BACKWARD | "n\a" | Sets motor directions to give backward movement |
| TURNL | "o\a" | Sets motor directions to give left rotation. |
| TURNR | "p\a" | Sets motor directions to give right rotation. |

Notes:	 all packets are string types.

\a denotes ASCII BEL character 0x07

<p> is a command parameter, replace with an appropriate value.

16-bit servo pulse widths must be in range 750-2400 (hardware reasons)

GETV uses costly floating point calculations. Do not use more often than needed.

Raspberry Pi 3

Board